4.6 DESIGN OF BEAMS:

4.6.1 BEAM:

Beam is a horizontal structural member which transmits loads horizontally along their length to the supports where the loads are generally transformed into vertical forces. The objective of Beams is to withstand vertical loads, shear forces and bending moments.

Depending on the types of supports the beams are classified as:

- Simply supported beam

- Cantilever beam

- Over hanging beam

- Continuous beam

- Fixed beam

- Propped cantilever beam

Code limitations:

In IS-456-2000-page No.48, the limitations and specifications for beam design are given as

Beams:

Tension reinforcement:

a) Minimum reinforcement -The minimum area of tension reinforcement shall be not less than that given by the following:

Asbd= 0.87fy

Where,

As= minimum area of tension reinforcement,

b = breadth of beam or the breadth of the web of T-beam.

d = effective depth, and

fy = characteristic strength of reinforcement in N/mm2

b) Maximum reinforcement – The maximum area of tension reinforcement shall not exceed 0.04bD.

Compression reinforcement:

The maximum area of compression reinforcement shall not exceed 0.04bD. Compression reinforcement in beams shall be enclosed by stirrups for effective lateral restraint.

Side face reinforcement:

Where the depth of the web in a beam exceeds 750mm, side face reinforcement shall be provided along the two faces. The total area of such reinforcement shall be not less than 0.1 percent of the web area and shall be distributed equally on two faces at a spacing not exceeding 300mm or web thickness whichever is less.

Transverse reinforcement in beams for shear and torsion:

The transverse reinforcement in beams shall be taken around the outer-most tension and compression bars. "In T-beams and I-beams, such reinforcement shall pass around longitudinal bars located close to the outer face of the flange

Maximum spacing of shear reinforcement:

The maximum spacing of shear reinforcement measured along the axis of the member shall not exceed 0.75d for vertical stirrups and d for inclined stirrups at 45o, where d is the effective depth of the section under consideration. In no case shall the spacing exceed 300mm.

Minimum shear reinforcement:

Minimum shear reinforcement in the form of stirrups shall be provided such that:

AsvbSv ≥ 0.40.87fy

Where,

Asv = total cross-sectional area of stirrup legs effective in shear.

Sv = stirrup Spacing along the Length of the member.

b = breadth of the beam or breadth of the web of flanged beam.

fy = characteristic strength of the stirrup reinforcement in N/mm2 which shall not be taken greater than 415N/mm2.

Where the maximum shear stress calculated is less than half the permissible value and in members of minor structural importance such as lintels. This provision need not be complied with.

Minimum reinforcement:

The mild steel reinforcement in either direction in slab shall not be less than 0.15percent of the total cross-sectional area. However, this value can be reduced to 0.12 percent when high strength deformed bars or welded wire fabric are used.

4.6.2 DESIGN OF CONTINUOUS BEAM IN FRAME A1-B1-C1:

ASSUMPTIONS:

Size of the beam

= 230X400mm

Effective cover = 30 mm

Grade of the concrete =M20

Grade of the steel = Fe 500

fck = 20 N/m2

fy = 500 N/m2

Effective depth (d)

= D - Effective cover

d = 400-30 = 370mm

Figure 4.5: Bending moment diagram {frame A1-B1-C1}

Code limits:

As per code if Mu is less than Mu,limit then the section is design as a singly reinforced section otherwise it should be design as doubly reinforced section.

Mu,limit = 0.36  xu maxd (1 - 0.42 xu maxd) bd2fck

Mu,limit = 0.36  0.46dd (1 - 0.42 0.46dd ) 230×3702 ×20

Mu,limit = 84.14 kN-m

The minimum and maximum area of steel in beam is given below [ It is given in page No.46of IS - 456-2000 in article 26.5.1.1.]

Ast (minimum)=0.85fy bd

=0.85500 ×230×370=144.67 mm2

Ast (maximum)=0.04bD

=0.04×230×400=3680 mm2

4.6.2.1 AREA OF THE REINFORCEMENT REQUIRED:

Let us consider BEAM A1-B1-C1:

Tension steel in span A1-B1:

M=6.89 kN-m

Mu=1.5×6.89 =10.34 kN-m

DEPTH REQUIRED:

Mu  =Mu,limit

Mu = 0.36  xu maxd (1 - 0.42 xu maxd) bd2fck

For 500 grade steel, xu max = 0.46d which is given in page No.70 of IS - 456-2000 in article38.1.] Here bending moment is used to check the thickness.

10.34×106 = 0.36  0.46dd (1 - 0.42 0.46dd ) 230×d2 ×20

d(required) = 130 mm

d(required)<d(provided)

So, the section is safe in bending.

Mu < Mu,limit

So, the section is designed as singly reinforced beam.

Mu= 0.87  fyAst  d (1- fyAstbd fck)

10.34×106= 0.87 × 500× Ast×370 (1- 500×Ast230×370× 20)

Ast=65.50 mm2

Ast(required)<Ast (minimum)

So, provide minimum Ast

Provide 2 bars of #10mm∅ bars {Ast(provided)=157.08 mm2}

Tension steel in span B1-C1:

M=9.58 kN-m

Mu=1.5×9.58 =14.37 kN-m< Mu,limit

So, the section is designed as singly reinforced beam.

Mu= 0.87  fyAst  d (1- fyAstbd fck)

14.37×106= 0.87 × 500× Ast×370 (1- 500×Ast230×370× 20)

Ast=91.76 mm2

Ast(required)<Ast (minimum)

So, provide minimum Ast

Provide 2 bars of #10mm∅ bars {Ast(provided)=157.08mm2}

Support steel at A1:

M=5.88kN-m

Mu=1.5×5.88 =8.82 kN-m < Mu,limit

So, the section is designed as singly reinforced beam.

Mu= 0.87  fyAst  d (1- fyAstbd fck)

8.82×106= 0.87 × 500× Ast×370 (1- 500×Ast230×370× 20)

Ast=57.74 mm2

𝐴𝑠𝑡(𝑟𝑒𝑞𝑢𝑖𝑟𝑒𝑑) < 𝐴𝑠𝑡 (𝑚𝑖𝑛𝑖𝑚𝑢𝑚)

Provide 2 bar of #10mm∅ bar {Ast(provided)=157.08 mm2}

Support steel at B1:

M=17.40 kN-m

Mu=1.5×17.40 =26.1 kN-m< Mu,limit

So, the section is designed as singly reinforced beam.

Mu= 0.87  fyAst  d (1- fyAstbd fck)

26.1×106= 0.87 × 500× Ast×370 (1- 500×Ast230×370× 20)

Ast=170.72 mm2

Provide 2 bar of #12mm∅ bar {Ast(provided)=226.20 mm2}

Support steel at C1:

M=8.05 kN-m

Mu=1.5×8.05 =12.08 kN-m< Mu,limit

So, the section is designed as singly reinforced beam.

Mu= 0.87  fyAst  d (1- fyAstbd fck)

12.08×106= 0.87 × 500× Ast×370 (1- 500×Ast230×370× 20)

Ast=76.78 mm2

Ast(required)<Ast (minimum)

So, provide minimum Ast

Provide 2 bars of #10mm∅ bars {Ast(provided)=157.08mm2}

4.6.2.2 Check for deflection:

For span A1-B1:

dreq= LBV ×MF

L = 2.8 m

The BV Value for Continuous Beam (IS – 456 – 2000, page no. 37)

BV = 26

Pt= Astbd ×100=157.08230 ×370 ×100= 0.18

From IS – 456 – 2000, page no. 38

fs=0.58 ×fy × Area of cross-section of steel requiredArea of cross-section of steel provided

fs=0.58 ×500 × 65.50157.08=120.92 N/mm2

From the graph, in IS – 456 – 2000, page no. 38

MF = 2

dreq= 280026 ×2=53.84<dprov=370mm

For span B1-C1:

dreq= LBV ×MF

L = 3.1 m

The BV Value for Continuous Beam (IS – 456 – 2000, page no. 37)

BV = 26

Pt= Astbd ×100=157.08230 ×370 ×100= 0.18

From IS – 456 – 2000, page no. 38

fs=0.58 ×fy × Area of cross-section of steel requiredArea of cross-section of steel provided

fs=0.58 ×500 × 91.76157.08=169.40 N/mm2

From the graph, in IS – 456 – 2000, page no. 38

MF = 2

dreq= 310026 ×2=59.61<dprov=370mm

4.6.2.3 Design of shear reinforcement in Slab beams:

Span A1-B1:

Joint A1:

V = 21.74 KN

Vu = 32.61 KN

The nominal shear stress in beam of uniform depth shall be obtained by the following equation.

τv=Vubd

=32.61 ×1000230×370=0.38 N/mm2

Pt=Astbd×100

=157.08230×370×100=0.18%

Under no circumstances, for shear reinforcement, the nominal shear stress in beam τv shall not exceed τc,max. [ It is given in page No.73 of IS - 456-2000 in Table No.20]

τc,max= 2.8 N/mm2

τv<τc,max

τcValue is given in page 73 of is 456: 2000. This value is depending on Pt value and grade of concrete. For our condition M20 grade and Pt=0.18 the value is given below

τc=0.30 N/mm2

τv>τc

As per IS 456 : 2000, when the  τv exeed τc value the shear reinforcement shall be provided.[ It is given in page No.72of IS - 456-2000 in article 40.4.]

Vus=Vu-τcbd

= 32.61 x 103 – 0.3 x 230 x 370 = 7.08 KN

Shear reinforcement shall be provided to carry a shear equal to Vu-τc bd. The strength of the shear reinforcement Vus for vertical stirrups is given below [IS 456 : 2000, p.no.73]

Vus=0.87  ffAsvd sv

Using 8mm, 2 leggedFe500 steel stirrups,                      Asv=2 × π4×82=100.53mm2

Sv=0.87×500×100.53×3707.08×1000=2285.35 mm

Code limits:

[ It is given in page No.47of IS - 456-2000 in article 26.5.1.5.]

It should be lesser of the following

- S=0.75d=0.75×370=277.5mm

- S=300mm

So, provide 8 mm dia stirrups at 275 mm c/c

Span A1-B1:

Joint B1:

V = 28.74 KN

Vu = 43.11 KN

The nominal shear stress in beam of uniform depth shall be obtained by the following equation.

τv=Vubd

=43.11 ×1000230×370=0.51 N/mm2

Pt=Astbd×100

=226.20230×370×100=0.27%

Under no circumstances, for shear reinforcement, the nominal shear stress in beam τv shall not exceed τc,max. [ It is given in page No.73 of IS - 456-2000 in Table No.20]

τc,max= 2.8 N/mm2

τv<τc,max

τcValue is given in page 73 of is 456: 2000. This value is depending on Pt value and grade of concrete. For our condition M20 grade and Pt=0.27 the value is given below

τc=0.37 N/mm2

τv>τc

As per IS 456 : 2000, when the  τv exeed τc value the shear reinforcement shall be provided.[ It is given in page No.72of IS - 456-2000 in article 40.4.]

Vus=Vu-τcbd

= 43.11 x 103 – 0.37 x 230 x 370 = 11.62 KN

Shear reinforcement shall be provided to carry a shear equal to Vu-τc bd. The strength of the shear reinforcement Vus for vertical stirrups is given below [IS 456 : 2000, p.no.73]

Vus=0.87  ffAsvd sv

Using 8mm, 2 legged Fe500 steel stirrups,                      Asv=2 × π4×82=100.53mm2

Sv=0.87×500×100.53×37011.62×1000=1392.45mm

Code limits:

[ It is given in page No.47of IS - 456-2000 in article 26.5.1.5.]

It should be lesser of the following

- S=0.75d=0.75×370=277.5mm

- S=300mm

So, provide 8 mm dia stirrups at 275 mm c/c

Span B1-C1:

Joint B1:

V = 31.79 KN

Vu = 47.69 KN

The nominal shear stress in beam of uniform depth shall be obtained by the following equation.

τv=Vubd

=47.69 ×1000230×370=0.56 N/mm2

Pt=Astbd×100

=226.20230×370×100=0.27%

Under no circumstances, for shear reinforcement, the nominal shear stress in beam τv shall not exceed τc,max. [ It is given in page No.73 of IS - 456-2000 in Table No.20]

τc,max= 2.8 N/mm2

τv<τc,max

τcValue is given in page 73 of is 456: 2000. This value is depending on Pt value and grade of concrete. For our condition M20 grade and Pt=0.27 the value is given below

τc=0.37 N/mm2

τv>τc

As per IS 456 : 2000, when the  τv exeed τc value the shear reinforcement shall be provided.[ It is given in page No.72of IS - 456-2000 in article 40.4.]

Vus=Vu-τcbd

= 47.69 x 103 – 0.37 x 230 x 370 = 16.20 KN

Shear reinforcement shall be provided to carry a shear equal to Vu-τc bd. The strength of the shear reinforcement Vus for vertical stirrups is given below [IS 456 : 2000, p.no.73]

Vus=0.87  ffAsvd sv

Using 8mm, 2 legged Fe500 steel stirrups,                      Asv=2 × π4×82=100.53mm2

Sv=0.87×500×100.53×37016.20×1000=998.78mm

Code limits:

[ It is given in page No.47of IS - 456-2000 in article 26.5.1.5.]

It should be lesser of the following

- S=0.75d=0.75×370=277.5mm

- S=300mm

So, provide 8 mm dia stirrups at 275 mm c/c

Span B1-C1:

Joint C1:

V = 25.75 KN

Vu = 38.63 KN

The nominal shear stress in beam of uniform depth shall be obtained by the following equation.

τv=Vubd

=38.63 ×1000230×370=0.45 N/mm2

Pt=Astbd×100

=157.08230×370×100=0.18%

Under no circumstances, for shear reinforcement, the nominal shear stress in beam τv shall not exceed τc,max. [ It is given in page No.73 of IS - 456-2000 in Table No.20]

τc,max= 2.8 N/mm2

τv<τc,max

τcValue is given in page 73 of is 456: 2000. This value is depending on Pt value and grade of concrete. For our condition M20 grade and Pt=0.18 the value is given below

τc=0.30 N/mm2

τv>τc

As per IS 456 : 2000, when the  τv exeed τc value the shear reinforcement shall be provided.[ It is given in page No.72of IS - 456-2000 in article 40.4.]

Vus=Vu-τcbd

= 38.63 x 103 – 0.30 x 230 x 370 = 13.10 KN

Shear reinforcement shall be provided to carry a shear equal to Vu-τc bd. The strength of the shear reinforcement Vus for vertical stirrups is given below [IS 456 : 2000, p.no.73]

Vus=0.87  ffAsvd sv

Using 8mm, 2 legged Fe500 steel stirrups,                      Asv=2 × π4×82=100.53mm2

Sv=0.87×500×100.53×37013.10×1000=1235.14 mm2

Code limits:

[ It is given in page No.47of IS - 456-2000 in article 26.5.1.5.]

It should be lesser of the following

- S=0.75d=0.75×370=277.5mm

- S=300mm

So, provide 8 mm dia stirrups at 275 mm c/c

Figure 4.6: Reinforcement details of beam A1-B1-C1.