Category 1: Project Scope & Planning (IS 875 & NBC 2016)

- What is the primary engineering objective of selecting the "Indiramma Illu" scheme for your mini-project?

- You mentioned a budget constraint of ₹5,00,000. As a civil engineer, how did you ensure your structural design did not inflate this cost?

- According to NBC 2016, under which exact group and subdivision does this building fall?

- Your gross plot area is 79.73 sq.m, but your net plot area is 54.82 sq.m. Explain the specific setback rules from G.O.Ms.No.168 you applied to arrive at this.

- Why did you provide a 1.5m setback at the front but only 0.5m on the sides?

- Explain the difference between Plinth Area and Carpet Area. What is the mandatory minimum built-up area for this government scheme?

- You restricted the overall depth of the slab to 100mm. Is this the minimum prescribed by IS 456? Justify your choice structurally.

- What is the minimum headroom clearance you maintained under the structural beams, and what does the code dictate?

- You planned for a road widening width of 4.5m from the center line. What happens to the structural footprint if the municipality decides to widen the road tomorrow?

- Why did you opt for load-bearing masonry for the walls instead of a fully framed RCC structure for a G+0 building?

Category 2: Load Calculations & Distribution

- You assumed the unit weight of RCC as 25 KN/m³ and brick masonry as 18 KN/m³. Which specific part of IS 875 gives you these values?

- For the roof slab, you took the live load as 1.5 KN/m², but for the floor slab, you took 2.0 KN/m². Explain this discrepancy.

- You stated you ignored Earthquake loads because Mahabubnagar is not in an earthquake-prone area. According to IS 1893, which seismic zone does Mahabubnagar fall under? Should it be entirely ignored?

- Explain the criteria for load distribution from a two-way slab to the supporting beams. Why does it form triangles and trapezoids?

- What are the specific formulas you used to calculate the equivalent Uniformly Distributed Load (UDL) on beams from a trapezoidal load distribution?

- How did you calculate the weight of the weathering course, and why is it factored into the dead load of the roof slab?

Category 3: Structural Analysis (Kani’s Method)

- You chose Kani’s Method over the Moment Distribution Method. What is the primary mathematical advantage of Kani’s Method for multi-storey frames?

- Define Rotation Factor. How did you calculate the sum of stiffness (∑ k) at joint B1?

- When calculating stiffness (k), you used 4EI/L. Under what support condition is this valid?

- In your Frame A1-B1-C1 analysis, you mentioned fixed end moments in all columns are zero. Why?

- What is a Displacement Factor, and why did you get a value of -0.5 for your columns in Storey 1?

- Explain the process of finding the "Joint Rotation" contribution in any given cycle of Kani's method.

- In Cycle 4 for Frame A1-B1-C1, how did you verify that the joint was in equilibrium to halt the iterations?

- How did you calculate the Final End Moments from the Fixed End Moments, Near End Rotation, and Far End Rotation?

- Looking at your Bending Moment diagram for Frame A1-B1-C1, why is the moment at support B1 significantly higher (22.30 KN-m) than at A1?

- If you had to analyze this same frame using STAAD.Pro, what differences would you expect in your final bending moment values, and why?

Category 4: Limit State Method & Slab Design (IS 456:2000)

- Why did you use the Limit State Method instead of the Working Stress Method for this project? What is the specific drawback of WSM?

- What are the partial safety factors for material strength of concrete and steel in the Limit State of Collapse?

- You chose M20 concrete and Fe500 steel. What does the "500" in Fe500 stand for, and what is its limiting depth of neutral axis ($x_{u, max}$)?

- For Slab S1, your Ly/Lx ratio is 1.19. What dictates that this must be designed as a two-way slab?

- How did you determine the effective depth of the slab along the shorter span versus the longer span? Why are they different?

- You calculated the factored bending moments using αx and αy coefficients. From which table in IS 456 did you extract these, and what boundary conditions do they represent?

- Your calculated required depth for the slab was 32.34mm, but you provided 100mm. Why the massive over-design?

- For the deflection check of the two-way slab, you used a Basic Value (BV) of 40. Where does this value come from?

- How did you calculate the Modification Factor (MF) for tension reinforcement in your deflection check?

Category 5: Beam Design & Shear Reinforcement

- In your continuous beam design, how did you ensure that Mu < Mu, limit}? What would you have to do if Mu exceeded Mu, limit?

- Explain the formula used to calculate the area of tension steel (Ast) for a singly reinforced section.

- For span A1-B1, you checked for shear. How is the nominal shear stress (τv) calculated?

- Your τv exceeded τc at joint B1. What is τc dependent on, and how did you extract it from Table 19 of IS 456?

- You provided 8mm dia 2-legged vertical stirrups at 275mm c/c. State the three IS code limit conditions for the maximum spacing of shear reinforcement.

- What is the significance of τc, max? What would you do if your nominal shear stress exceeded τc, max?

Category 6: Column Design (Uni-axial & Bi-axial Bending)

- You designed Column A1 for bi-axial bending. Define bi-axial bending and explain how it differs from uni-axial bending in Column B1.

- You assumed a reinforcement percentage (p) of 1.17% for Column A1. What is the minimum and maximum percentage of steel allowed in a column?

- Explain the interaction equation (Bresler's empirical formulation) used to check the safety of a bi-axially loaded column.

- In utilizing SP-16 Chart 48, you used the parameters Pu / fck . bD and Mu fckbD2. What exactly do these non-dimensional parameters represent?

- How did you determine the pitch (spacing) of the lateral ties for the columns? What three criteria did you check?

Category 7: Footing & Lintel Design

- For Footing A1, your Safe Bearing Capacity (SBC) is 200 KN/m². How does SBC impact the size of your footing?

- Explain the exact location of the critical section for One-Way Shear in an isolated footing.

- Explain the exact location of the critical section for Two-Way (Punching) Shear in an isolated footing.

- In your lintel design, you assumed a 60o load dispersion angle. Why didn't you consider the full rectangular load of the masonry wall above the opening?