Analysis of Building Frames (Using Kani’s Method)

Gasper Kani, a German engineer, developed another distribution procedure based on slope deflection equations. This method is very useful for the analysis of multistorey frames. The greatest advantage of this method is, even if a mistake is committed in distribution in one of the cycles, it converges finally to the correct answer. Even today, many practising engineers who are not familiar with computer methods, use Kani's method for the analysis of 3 to 4 storey building frames.

This method is first explained for structures with fixed ends. Then, the modifications to handle simply supported and overhanging ends are discussed. Analysis of symmetric frames making use of the symmetry is also explained, after which an analysis of general frames is taken up.

Figure 1: Beam Layout

Figure 2: Frames.

Analysis of Frame A1-B1-C1

Step 1: Fixed End Moments

MFA1B1 = -WL212 = -18.03 × 2.8212 = -11.78 KN-m

MFB1A1 = WL212 = 18.03 × 2.8212 = +11.78 KN-m

MFB1C1 = -WL212 = -18.56 × 3.1212 = -14.86 KN-m

MFC1B1 = WL212 = 18.56 × 3.1212 = +14.86 KN-m

Fixed end moments in All the columns are zero.

i.e,

MFA1A1՛ = MFA1՛A1 = MFB1B1՛ = MFB1՛B1 = MFC1C1՛ = MFC1՛C1 = 0

Step 2: Rotation Factor R.F = -12 kΣk , k= 4EIL

| Joints | Members | Stiffness (k) | Σk | R.F |
| --- | --- | --- | --- | --- |
| A1 | A1B1 | 1.428EI | 2.909EI | -0.245 |
|  | A1A1՛ | 1.481EI |  | -0.255 |
| B1 | B1A1 | 1.428EI | 4.200EI | -0.170 |
|  | B1C1 | 1.290EI |  | -0.154 |
|  | B1B1՛ | 1.481EI |  | -0.176 |
| C1 | C1B1 | 1.290EI | 2.771EI | -0.233 |
|  | C1C1՛ | 1.481EI |  | -0.267 |

Stiffness (k)      k=4EIL

KA1B1 = 4EI2.8 = 1.428EI

KA1A1՛ = 4EI2.7 = 1.481EI

KB1A1 = 4EI2.8 = 1.428EI

KB1C1 = 4EI3.1 = 1.290EI

KB1B1՛ = 4EI2.7 = 1.481EI

KC1B1 = 4EI3.1 = 1.290EI

KC1C1՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Joints

A1: 1.428EI + 1.481EI = 2.909EI

B1: 1.428EI + 1.290EI + 1.481EI = 4.200EI

C1: 1.290EI + 1.481EI = 2.771EI

Rotation Factor R.F = -12 kΣk

R.FA1B1 = -12 1.428EI2.909EI = -0.245

R.FA1A1՛ = -12 1.481EI2.909EI = -0.255

R.FB1A1 = -12 1.428EI4.200EI = -0.170

R.FB1C1 = -12 1.290EI4.200EI = -0.154

R.FB1B1՛ = -12 1.481EI4.200EI = -0.176

R.FC1B1 = -12 1.290EI2.771EI = -0.233

R.FC1C1՛ = -12 1.481EI2.771EI = -0.267

Step 3: Displacement Factor D.F = -32 kΣk

| Storey | Members | k | Σk | D.F |
| --- | --- | --- | --- | --- |
| 1 | A1A1՛ | 1.481EI | 4.444EI | -0.5 |
|  | B1B1՛ | 1.481EI |  | -0.5 |
|  | C1C1՛ | 1.481EI |  | -0.5 |

Stiffness (k)      k=4EIL

KA1A1՛ = 4EI2.7 = 1.481EI

KB1B1՛ = 4EI2.7 = 1.481EI

KC1C1՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Storey

∑k = 1.481EI + 1.481EI + 1.481EI = 4.444EI

Displacement Factor D.F = -32 kΣk

D.FA1A1՛ = -32 1.481EI4.444EI = -0.5

D.FB1B1՛ = -32 1.481EI4.444EI = -0.5

D.FC1C1՛ = -32 1.481EI4.444EI = -0.5

Rotation and Displacement Contribution

Cycle 1

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA1B1 = -0.245 × [-11.78 + 0 + 0] = 2.886

MA1A1՛ = -0.255 × [-11.78 + 0 + 0] = 3.003

MB1A1 = -0.170 × [-3.08 + 2.886 + 0] = 0.033

MB1C1 = -0.154 × [-3.08 + 2.886 + 0] = 0.030

MB1B1՛ = -0.176 × [-3.08 + 2.886 + 0] = 0.034

MC1B1 = -0.233 × [14.86 + 0.030 + 0] = -3.469

MC1C1՛ = -0.267 × [14.86 + 0.030 + 0] = -3.976

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA1A1՛ + MB1B1՛ + MC1C1՛ = -0.5 × (3.003 + 0.034 + (-3.976)) = 0.470

Cycle 2

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA1B1 = -0.245 × [-11.78 + 0.470 + 0.033] = 2.763

MA1A1՛ = -0.255 × [-11.78 + 0.470 + 0.033] = 2.876

MB1A1 = -0.170 × [-3.08 + 0.470 + 2.763 + (-3.469)] = 0.564

MB1C1 = -0.154 × [-3.08 + 0.470 + 2.763 + (-3.469)] = 0.510

MB1B1՛ = -0.176 × [-3.08 + 0.470 + 2.763 + (-3.469)] = 0.584

MC1B1 = -0.233 × [14.86 + 0.470 + 0.510] = -3.691

MC1C1՛ = -0.267 × [14.86 + 0.470 + 0.510] = -4.230

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA1A1՛ + MB1B1՛ + MC1C1՛ = -0.5 × (2.876 + 0.584 + (-4.230)) = 0.385

Cycle 3

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA1B1 = -0.245 × [-11.78 + 0.385 + 0.564] = 2.654

MA1A1՛ = -0.255 × [-11.78 + 0.385 + 0.564] = 2.762

MB1A1 = -0.170 × [-3.08 + 0.385 + 2.654 + (-3.691)] = 0.634

MB1C1 = -0.154 × [-3.08 + 0.385 + 2.654 + (-3.691)] = 0.575

MB1B1՛ = -0.176 × [-3.08 + 0.385 + 2.654 + (-3.691)] = 0.657

MC1B1 = -0.233 × [14.86 + 0.385 + 0.575] = -3.686

MC1C1՛ = -0.267 × [14.86 + 0.385 + 0.575] = -4.224

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA1A1՛ + MB1B1՛ + MC1C1՛ = -0.5 × (2.762 + 0.657 + (-4.224)) = 0.4025

Cycle 4

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA1B1 = -0.245 × [-11.78 + 0.4025 + 0.634] = 2.632

MA1A1՛ = -0.255 × [-11.78 + 0.4025 + 0.634] = 2.740

MB1A1 = -0.170 × [-3.08 + 0.4025 + 2.632 + (-3.686)] = 0.634

MB1C1 = -0.154 × [-3.08 + 0.4025 + 2.632 + (-3.686)] = 0.575

MB1B1՛ = -0.176 × [-3.08 + 0.4025 + 2.632 + (-3.686)] = 0.657

MC1B1 = -0.233 × [14.86 + 0.4025 + 0.575] = -3.690

MC1C1՛ = -0.267 × [14.86 + 0.4025 + 0.575] = -4.230

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA1A1՛ + MB1B1՛ + MC1C1՛ = -0.5 × (2.740 + 0.657 + (-4.230)) = 0.4165

Step 4: Final End Moments

Mij = FEM + 2×Mij (Near end Rotation) + Mji (Far end Rotation) +M՛՛ (Displacement)

Beams (No Sway M՛՛= 0)

MA1B1 = -11.78 + 2(2.632) + 0.634 = -5.88

MB1A1 = 11.78 + 2(0.634) + 2.632 = 15.68

MB1C1 = -14.86+ 2(0.575) + (-3.690) = -17.40

MC1B1 = 14.86 + 2(-3.690) + 0.575 = 8.05

Column (With Sway M՛՛= 0.4165)

MA1A1՛ = 0 + 2(2.740) + 0 + 0.4165 = 5.89

MA1՛A1 = 0 + 2(0) + 2.740 + 0.4165 = 3.15

MB1B1՛ = 0 + 2(0.657) + 0 + 0.4165 = 1.73

MB1՛B1 = 0 + 2(0) + 0.657 + 0.4165 = 1.07

MC1C1՛ = 0 + 2(-4.230) + 0 + 0.4165 = -8.04

MC1՛C1 = 0 + 2(0) + (-4.230) + 0.4165 = -3.81

Equilibrium Check

Joint A1 = MA1B1 + MA1A1՛ = -5.88 + 5.89 = 0.01 ≈ 0

Joint A1 = MB1A1 + MB1C1՛ + MB1B1՛ = 15.68 + (-17.40) + 1.73 = 0.01 ≈ 0

Joint A1 = MC1B1 + MC1C1՛ = 8.05 – 8.04 = 0.01 ≈ 0

CALCULATION BENDING MOMENT

BMA1B1 = WL28 = 18.03 ×2.828 = 17.67KN-m

BMB1C1 = WL28 = 18.56 ×3.128 = 22.30KN-m

BENDING MOMENT DIAGRAM

CALCULATION OF REACTIONS

SPAN A1-B1

Length (L) = 2.8m, UDL (W) = 18.03KN/m

Simply Supported Reactions:

Rtotal = WL2 = 18.03 ×2.82 = 25.24KN

Reaction Due to End Moments:

Rmoments = MB1A1- MA1B1L = 15.68- 5.882.8 = 3.50KN

Final Reactions:

RA1 = Rtotal – Rmoments = 25.24 – 3.50 = 21.74KN

RB1(Left) = Rtotal + Rmoments = 25.24 + 3.50 = 28.74KN

SPAN B1-C1

Length (L) = 3.1m, UDL (W) = 18.56KN/m

Simply Supported Reactions:

Rtotal = WL2 = 18.56 ×3.12 = 28.77KN

Reaction Due to End Moments:

Rmoments = MB1C1- MC1B1)L = 17.40- 8.053.1 = 3.02KN

Final Reactions:

RB1(Right) = Rtotal + Rmoments = 28.77 + 3.02 = 31.79KN

RC1= Rtotal – Rmoments = 28.77 – 3.02 = 25.75KN

Total Reactions:

Reaction at Column A1 (RA1) = 21.74KN

Reaction at Column B1 (RB1) = RB1(Left) + RB1(Right) = 28.74 + 31.79 = 60.53KN

Reaction at Column C1 (RC1) = 25.75KN