Analysis of Building Frames (Using Kani’s Method)

Gasper Kani, a German engineer, developed another distribution procedure based on slope deflection equations. This method is very useful for the analysis of multistorey frames. The greatest advantage of this method is, even if a mistake is committed in distribution in one of the cycles, it converges finally to the correct answer. Even today, many practising engineers who are not familiar with computer methods, use Kani's method for the analysis of 3 to 4 storey building frames.

This method is first explained for structures with fixed ends. Then, the modifications to handle simply supported and overhanging ends are discussed. Analysis of symmetric frames making use of the symmetry is also explained, after which an analysis of general frames is taken up.

Figure 1: Beam Layout

Figure 2: Frames.

Analysis of Frame A2-B2-C2

Step 1: Fixed End Moments

MFOverhanging =   WL22 =  4.16 ×1.9222 = 7.67 KN-m

MFA2B2 = -WL212 = -16.84 × 2.8212 = -11.00 KN-m

MFB2A2 = WL212 = -16.84 × 2.8212 = +11.00 KN-m

MFB2C2 = -WL212 = -17.92 × 3.1212 = -14.35 KN-m

MFC2B2 = WL212 = 17.92 × 3.1212 = +14.35 KN-m

Fixed end moments in All the columns are zero.

i.e,

MFA2A2՛ = MFA2՛A2 = MFB2B2՛ = MFB2՛B2 = MFC2C2՛ = MFC2՛C2 = 0

Step 2: Rotation Factor R.F = -12 kΣk , k= 4EIL

| Joints | Members | Stiffness (k) | Σk | R.F |
| --- | --- | --- | --- | --- |
| A2 | A2B2 | 1.428EI | 2.909EI | -0.245 |
|  | A2A2՛ | 1.481EI |  | -0.255 |
| B2 | B2A2 | 1.428EI | 4.200EI | -0.170 |
|  | B2C2 | 1.290EI |  | -0.154 |
|  | B2B2՛ | 1.481EI |  | -0.176 |
| C2 | C2B2 | 1.290EI | 2.771EI | -0.233 |
|  | C2C2՛ | 1.481EI |  | -0.267 |

Stiffness (k)      k=4EIL

KA2B2 = 4EI2.8 = 1.428EI

KA2A2՛ = 4EI2.7 = 1.481EI

KB2A2 = 4EI2.8 = 1.428EI

KB2C2 = 4EI3.1 = 1.290EI

KB2B2՛ = 4EI2.7 = 1.481EI

KC2B2 = 4EI3.1 = 1.290EI

KC2C2՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Joints

A2: 1.428EI + 1.481EI = 2.909EI

B2: 1.428EI + 1.290EI + 1.481EI = 4.200EI

C2: 1.290EI + 1.481EI = 2.771EI

Rotation Factor R.F = -12 kΣk

R.FA2B2 = -12 1.428EI2.909EI = -0.245

R.FA2A2՛ = -12 1.481EI2.909EI = -0.255

R.FB2A2 = -12 1.428EI4.200EI = -0.170

R.FB2C2 = -12 1.290EI4.200EI = -0.154

R.FB2B2՛ = -12 1.481EI4.200EI = -0.176

R.FC2B2 = -12 1.290EI2.771EI = -0.233

R.FC2C2՛ = -12 1.481EI2.771EI = -0.267

Step 3: Displacement Factor D.F = -32 kΣk

| Storey | Members | k | Σk | D.F |
| --- | --- | --- | --- | --- |
| 1 | A2A2՛ | 1.481EI | 4.444EI | -0.5 |
|  | B2B2՛ | 1.481EI |  | -0.5 |
|  | C2C2՛ | 1.481EI |  | -0.5 |

Stiffness (k)      k=4EIL

KA2A2՛ = 4EI2.7 = 1.481EI

KB2B2՛ = 4EI2.7 = 1.481EI

KC2C2՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Storey

∑k = 1.481EI + 1.481EI + 1.481EI = 4.444EI

Displacement Factor D.F = -32 kΣk

D.FA2A2՛ = -32 1.481EI4.444EI = -0.5

D.FB2B2՛ = -32 1.481EI4.444EI = -0.5

D.FC2C2՛ = -32 1.481EI4.444EI = -0.5

Rotation and Displacement Contribution

starting FEM at Joint A2 ∑FEM will be the sum of the overhang and the beam:

∑FEMA2 = 7.67 + (-11.00) = -3.33KN-m

Cycle 1

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA2B2 = -0.245 × [-3.33 + 0 + 0] = 0.816

MA2A2՛ = -0.255 × [-3.33 + 0 + 0] = 0.849

MB2A2 = -0.170 × [-3.35 + 0.816 + 0] = 0.431

MB2C2 = -0.154 × [-3.35 + 0.816 + 0] = 0.390

MB2B2՛ = -0.176 × [-3.35 + 0.816 + 0] = 0.446

MC2B2 = -0.233 × [14.35 + 0.390 + 0] = -3.434

MC2C2՛ = -0.267 × [14.35 + 0.390 + 0] = -3.936

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA2A2՛ + MB2B2՛ + MC2C2՛ = -0.5 × (0.849 + 0.446 + (-3.936)) = 1.321

Cycle 2

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA2B2 = -0.245 × [-3.33 + 1.321 + 0.431] = 0.387

MA2A2՛ = -0.255 × [-3.33 + 1.321 + 0.431] = 0.402

MB2A2 = -0.170 × [-3.35 + 1.321 + 0.387 + (-3.434)] = 0.863

MB2C2 = -0.154 × [-3.35 + 1.321 + 0.387 + (-3.434)] = 0.782

MB2B2՛ = -0.176 × [-3.35 + 1.321 + 0.387 + (-3.434)] = 0.893

MC2B2 = -0.233 × [14.35 + 1.321 + 0.782] = -3.834

MC2C2՛ = -0.267 × [14.35 + 1.321 + 0.782] = -4.393

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA2A2՛ + MB2B2՛ + MC2C2՛ = -0.5 × (0.402 + 0.893 + (-4.393)) = 1.549

Cycle 3

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA2B2 = -0.245 × [-3.33 + 1.549 + 0.863] = 0.225

MA2A2՛ = -0.255 × [-3.33 + 1.549 + 0.863] = 0.234

MB2A2 = -0.170 × [-3.35 + 1.549 + 0.225 + (-3.834)] = 0.920

MB2C2 = -0.154 × [-3.35 + 1.549 + 0.225 + (-3.834)] = 0.833

MB2B2՛ = -0.176 × [-3.35 + 1.549 + 0.225 + (-3.834)] = 0.952

MC2B2 = -0.233 × [14.35 + 1.549 + 0.833] = -3.899

MC2C2՛ = -0.267 × [14.35 + 1.549 + 0.833] = -4.467

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA2A2՛ + MB2B2՛ + MC2C2՛ = -0.5 × (0.234 + 0.952 + (-4.467)) = 1.641

Cycle 4

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MA2B2 = -0.245 × [-3.33 + 1.641 + 0.920] = 0.188

MA2A2՛ = -0.255 × [-3.33 + 1.641 + 0.920] = 0.196

MB2A2 = -0.170 × [-3.35 + 1.641 + 0.188 + (-3.899)] = 0.921

MB2C2 = -0.154 × [-3.35 + 1.641 + 0.188 + (-3.899)] = 0.835

MB2B2՛ = -0.176 × [-3.35 + 1.641 + 0.188 + (-3.899)] = 0.954

MC2B2 = -0.233 × [14.35 + 1.641 + 0.835] = -3.920

MC2C2՛ = -0.267 × [14.35 + 1.641 + 0.835] = -4.493

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MA2A2՛ + MB2B2՛ + MC2C2՛ = -0.5 × (0.196 + 0.954 + (-4.493)) = 1.672

Step 4: Final End Moments

Mij = FEM + 2×Mij (Near end Rotation) + Mji (Far end Rotation) +M՛՛ (Displacement)

Beams (No Sway M՛՛= 0) “for Joint A2, the Overhang Moment is a fixed constant of +7.67 KN-m”

MA2B2 = -11.00 + 2(0.188) + 0.921 = -9.70

MB2A2 = 11.00 + 2(0.921) + 0.188 = 13.03

MB2C2 = -14.35+ 2(0.835) + (-3.920) = -16.60

MC2B2 = 14.35 + 2(-3.920) + 0.835 = 7.35

Column (With Sway M՛՛= 1.672)

MA2A2՛ = 0 + 2(0.196) + 0 + 1.672 = 2.06

MA2՛A2 = 0 + 2(0) + 0.196 + 1.672 = 1.87

MB2B2՛ = 0 + 2(0.954) + 0 + 1.672 = 3.58

MB2՛B2 = 0 + 2(0) + 0.954 + 1.672 = 2.63

MC2C2՛ = 0 + 2(-4.493) + 0 + 1.672 = -7.31

MC2՛C2 = 0 + 2(0) + (-4.493) + 1.672 = -2.82

Equilibrium Check

“To check Joint A2, we must include that +7.67 overhanging moment along with the beam and column.”

Joint A2 = Moverhanging + MA2B2 + MA2A2՛ = 7.67 + (-9.70) + 2.06 = 0.03 ≈ 0

Joint B2 = MB2A2 + MB2C2՛ + MB2B2՛ = 13.03 + (-16.60) + 3.58 = 0.01 ≈ 0

Joint C2 = MC2B2 + MC2C2՛ = 7.35 – 7.31 = 0.04 ≈ 0

CALCULATION BENDING MOMENT

“Since the overhang is a cantilever, it doesn't have a simply supported "sagging" mid-span moment. Its moment is purely negative (hogging), starting at 0 at the free end and curving to 7.67 KN-m at support A2.”

BMA2B2 = WL28 = 16.84 ×2.828 = 16.50KN-m

BMB2C2 = WL28 = 17.92 ×3.128 = 21.53KN-m

BENDING MOMENT DIAGRAM

CALCULATION OF REACTIONS

From Overhang (Direct load transfer):

Roverhang = w × L = 4.16 × 1.92 = 7.99KN

For Span A2B2

Simply Supported Reactions:

Rtotal @ A2B2= WL2 = 16.84 ×2.82 = 23.58KN

Reaction Due to End Moments: (Net clockwise moment means left support gets less load)

Rmoment @ A2B2=  MA2B2- MB2A2L =  -9.70 + 13.032.8 = -1.19KN

Reaction Due to End Moments: (Net clockwise moment means right support gets more load)

Rmoment @ A2B2=  MA2B2- MB2A2L =  +9.70 + 13.032.8 = +1.19KN

For Span B2C2

Simply Supported Reactions:

Rtotal @ B2C2= WL2 = 17.92 ×3.12 = 27.78KN

Reaction Due to End Moments: (Net clockwise moment means right support gets less load)

Rmoment @ B2C2=  MB2C2- MC2B2L =  -16.60 + 7.353.1 = -2.98KN

Reaction Due to End Moments: (Net clockwise moment means left support gets more load)

Rmoment @ B2C2=  MB2C2- MC2B2L =  16.60 + 7.353.1 = +2.98KN

Final Reactions:

RA2 = Roverhang + Rtotal @ A2B2+ Rmoments @ A2B2(Left Support)

RA2 =7.99 + 23.58 + (-1.19) =  30.38KN

R B2 = (Rtotal @ A2B2+ Rmoments @ A2B2(Right Support)) + (Rtotal @ B2C2+ Rmoments @ B2C2(Left Support)) =

R B2 = (23.58 + 1.19) + (27.78 + 2.98) = 55.53

R C2 = Rtotal @ B2C2+ Rmoments @ B2C2(Right Support) =

R C2 = 27.78 + (-2.98) = 24.80 KN