Analysis of Building Frames (Using Kani’s Method)

Gasper Kani, a German engineer, developed another distribution procedure based on slope deflection equations. This method is very useful for the analysis of multistorey frames. The greatest advantage of this method is, even if a mistake is committed in distribution in one of the cycles, it converges finally to the correct answer. Even today, many practising engineers who are not familiar with computer methods, use Kani's method for the analysis of 3 to 4 storey building frames.

This method is first explained for structures with fixed ends. Then, the modifications to handle simply supported and overhanging ends are discussed. Analysis of symmetric frames making use of the symmetry is also explained, after which an analysis of general frames is taken up.

Figure 1: Beam Layout

Figure 2: Frames.

Analysis of Frame A3-B3-C3

Step 1: Fixed End Moments

MFOverhanging =    =   = 14.16 KN-m

MFA3B3 =  =  = -11.77 KN-m

MFB3A3 =  =  = +11.77 KN-m

MFB3C3 =  =  = -14.86 KN-m

MFC3B3 =  =  = +14.86 KN-m

Fixed end moments in All the columns are zero.

i.e,

MFA3A3՛ = MFA3՛A3 = MFB3B3՛ = MFB3՛B3 = MFC3C3՛ = MFC3՛C3 = 0

Step 2: Rotation Factor ,

| Joints | Members | Stiffness () |  | R.F |
| --- | --- | --- | --- | --- |
| A3 | A3B3 | 1.428EI | 2.909EI | -0.245 |
|  | A3A3՛ | 1.481EI |  | -0.255 |
| B3 | B3A3 | 1.428EI | 4.200EI | -0.170 |
|  | B3C3 | 1.290EI |  | -0.154 |
|  | B3B3՛ | 1.481EI |  | -0.176 |
| C3 | C3B3 | 1.290EI | 2.771EI | -0.233 |
|  | C3C3՛ | 1.481EI |  | -0.267 |

Stiffness ()

KA3B3 =  = 1.428EI

KA3A3՛ =  = 1.481EI

KB3A3 =  = 1.428EI

KB3C3 =  = 1.290EI

KB3B3՛ =  = 1.481EI

KC3B3 =  = 1.290EI

KC3C3՛ =  = 1.481EI

Sum of Stiffness ∑ at Joints

A3: 1.428EI + 1.481EI = 2.909EI

B3: 1.428EI + 1.290EI + 1.481EI = 4.200EI

C3: 1.290EI + 1.481EI = 2.771EI

Rotation Factor

R.FA3B3 =  = -0.245

R.FA3A3՛ = = -0.255

R.FB3A3 =  = -0.170

R.FB3C3 =  = -0.154

R.FB3B3՛ =  = -0.176

R.FC3B3 =  = -0.233

R.FC3C3՛ =  = -0.267

Step 3: Displacement Factor

| Storey | Members |  |  | D.F |
| --- | --- | --- | --- | --- |
| 1 | A3A3՛ | 1.481EI | 4.444EI | -0.5 |
|  | B3B3՛ | 1.481EI |  | -0.5 |
|  | C3C3՛ | 1.481EI |  | -0.5 |

Stiffness ()

KA3A3՛ =  = 1.481EI

KB3B3՛ =  = 1.481EI

KC3C3՛ =  = 1.481EI

Sum of Stiffness ∑ at Storey

∑ = 1.481EI + 1.481EI + 1.481EI = 4.444EI

Displacement Factor

D.FA3A3՛ = = -0.5

D.FB3B3՛ =  = -0.5

D.FC3C3՛ =  = -0.5

Rotation and Displacement Contribution

starting FEM at Joint A3 ∑FEM will be the sum of the overhang and the beam:

∑FEMA3 = 14.16 + (-11.77) = 2.39KN-m

Cycle 1

Joint Rotation

MA3B3 = -0.245 × [2.39 + 0 + 0] = -0.586

MA3A3՛ = -0.255 × [2.39+ 0 + 0] = -0.609

MB3A3 = -0.170 × [-3.09 + (-0.586) + 0] = 0.625

MB3C3 = -0.154 × [-3.09 + (-0.586) + 0] = 0.566

MB3B3՛ = -0.176 × [-3.09 + (-0.586) + 0] = 0.647

MC3B3 = -0.233 × [14.86 + 0.566 + 0] = -3.594

MC3C3՛ = -0.267 × [14.86 + 0.566 + 0] = -4.119

Displacement Contribution

For All Columns

MA3A3՛ + MB3B3՛ + MC3C3՛ = -0.5  (-0.609 + 0.647 + (-4.119)) = 2.041

Cycle 2

Joint Rotation

MA3B3 = -0.245 × [2.39 + 2.041 + 0.625] = -1.239

MA3A3՛ = -0.255 × [2.39 + 2.041 + 0.625] = -1.289

MB3A3 = -0.170 × [-3.09 + 2.041 + (-1.239) + (-3.594)] = 1.000

MB3C3 = -0.154 × [-3.09 + 2.041 + (-1.239) + (-3.594)] = 0.906

MB3B3՛ = -0.176 × [-3.09 + 2.041 + (-1.239) + (-3.594)] = 1.035

MC3B3 = -0.233 × [14.86 + 2.041 + 0.906] = -4.149

MC3C3՛ = -0.267 × [14.86 + 2.041 + 0.906] = -4.754

Displacement Contribution

For All Columns

MA3A3՛ + MB3B3՛ + MC3C3՛ = -0.5  (-1.289 + 1.035 + (-4.754)) = 2.504

Cycle 3

Joint Rotation

MA3B3 = -0.245 × [2.39 + 2.504 + 1.000] = -1.444

MA3A3՛ = -0.255 × [2.39 + 2.504 + 1.000] = -1.503

MB3A3 = -0.170 × [-3.09 + 2.504 + (-1.444) + (-4.149)] = 1.050

MB3C3 = -0.154 × [-3.09 + 2.504 + (-1.444) + (-4.149)] = 0.952

MB3B3՛ = -0.176 × [-3.09 + 2.504 + (-1.444) + (-4.149)] = 1.088

MC3B3 = -0.233 × [14.86 + 2.504 + 0.952] = -4.268

MC3C3՛ = -0.267 × [14.86 + 2.504 + 0.952] = -4.890

Displacement Contribution

For All Columns

MA3A3՛ + MB3B3՛ + MC3C3՛ = -0.5  (-1.503 + 1.088 + (-4.890)) = 2.653

Cycle 4

Joint Rotation

MA3B3 = -0.245 × [2.39 + 2.653 + 1.050] = -1.493

MA3A3՛ = -0.255 × [2.39 + 2.653 + 1.050] = -1.554

MB3A3 = -0.170 × [-3.09 + 2.653 + (-1.493) + (-4.268)] = 1.054

MB3C3 = -0.154 × [-3.09 + 2.653 + (-1.493) + (-4.268)] = 0.954

MB3B3՛ = -0.176 × [-3.09 + 2.653 + (-1.493) + (-4.268)] = 1.091

MC3B3 = -0.233 × [14.86 + 2.653 + 0.954] = -4.303

MC3C3՛ = -0.267 × [14.86 + 2.653 + 0.954] = -4.931

Displacement Contribution

For All Columns

MA3A3՛ + MB3B3՛ + MC3C3՛ = -0.5  (-1.554 + 1.091 + (-4.931)) = 2.697

Step 4: Final End Moments

Mij = FEM + 2×Mij (Near end Rotation) + Mji (Far end Rotation) +M՛՛ (Displacement)

Beams (No Sway M՛՛= 0) “for Joint A3, the Overhang Moment is a fixed constant of +14.16 KN-m”

MA3B3 = -11.77 + 2(-1.493) + 1.054 = -13.70 KN-m

MB3A3 = 11.77 + 2(1.054) + (-1.493) = 12.39 KN-m

MB3C3 = -14.86+ 2(0.954) + (-4.303) = -17.26 KN-m

MC3B3 = 14.86 + 2(-4.303) + 0.954 = 7.21 KN-m

Column (With Sway M՛՛= 2.697)

MA3A3՛ = 0 + 2(-1.554) + 0 + 2.697 = -0.41 KN-m

MA3՛A3 = 0 + 2(0) + (-1.554) + 2.697 = 1.14 KN-m

MB3B3՛ = 0 + 2(1.091) + 0 + 2.697 = 4.88 KN-m

MB3՛B3 = 0 + 2(0) + 1.091 + 2.697 = 3.79 KN-m

MC3C3՛ = 0 + 2(-4.931) + 0 + 2.697 = -7.17 KN-m

MC3՛C3 = 0 + 2(0) + (-4.931) + 2.697 = -2.23 KN-m

Equilibrium Check

“To check Joint A3, we must include that +14.16 overhanging moment along with the beam and column.”

Joint A3 = Moverhanging + MA3B3 + MA3A3՛ = 14.16 + (-13.70) + (-0.41) = 0.05 ≈ 0

Joint B3 = MB3A3 + MB3C3՛ + MB3B3՛ = 12.39 + (-17.26) + 4.88 = 0.01 ≈ 0

Joint C3 = MC3B3 + MC3C3՛ = 7.21 + (-7.17) = 0.04 ≈ 0

CALCULATION BENDING MOMENT

“Since the overhang is a cantilever, it doesn't have a simply supported "sagging" mid-span moment. Its moment is purely negative (hogging), starting at 0 at the free end and curving to 14.16 KN-m at support A3.”

BMA3B3 =  =  = 17.65KN-m

BMB3C3 =  =  = 22.30KN-m

BENDING MOMENT DIAGRAM

CALCULATION OF REACTIONS

From Overhang (Direct load transfer):

Roverhang = w × L = 7.68 × 1.92 = 14.75KN

For Span A3B3

Simply Supported Reactions:

Rtotal @ A3B3=  =  = 25.21KN

Reaction Due to End Moments: (Net clockwise moment means left support gets less load)

Rmoment @ A3B3=   =   = +0.47KN

Reaction Due to End Moments: (Net clockwise moment means right support gets more load)

Rmoment @ A3B3=   =   = -0.47KN

For Span B3C3

Simply Supported Reactions:

Rtotal @ B3C3 =  =  = 28.77KN

Reaction Due to End Moments: (Net clockwise moment means left support gets more load)

Rmoment @ B3C3=   =   = +3.24KN

Reaction Due to End Moments: (Net clockwise moment means right support gets less load)

Rmoment @ B3C3=   =   = -3.24KN

Final Reactions:

RA3 = Roverhang + Rtotal @ A3B3 + Rmoments @ A3B3(Left Support)

RA3 = 14.75 + 25.21 + 0.47 =  40.43KN

R B3 = (Rtotal @ A3B3+ Rmoments @ A3B3(Right Support)) + (Rtotal @ B3C3+ Rmoments @ B3C3(Left Support)) =

R B3 = (25.21 + (-0.47)) + (28.77 + 3.24) = 56.75KN

R C3 = Rtotal @ B3C3+ Rmoments @ B3C3(Right Support) =

R C3 = 28.77 + (-3.24) = 25.53KN