Analysis of Building Frames (Using Kani’s Method)

Gasper Kani, a German engineer, developed another distribution procedure based on slope deflection equations. This method is very useful for the analysis of multistorey frames. The greatest advantage of this method is, even if a mistake is committed in distribution in one of the cycles, it converges finally to the correct answer. Even today, many practising engineers who are not familiar with computer methods, use Kani's method for the analysis of 3 to 4 storey building frames.

This method is first explained for structures with fixed ends. Then, the modifications to handle simply supported and overhanging ends are discussed. Analysis of symmetric frames making use of the symmetry is also explained, after which an analysis of general frames is taken up.

Figure 1: Beam Layout

Figure 2: Frames.

Analysis of Frame B1-B2-B3

Step 1: Fixed End Moments

MFB1B2 = -WL212 = -18.65 × 3.33212 = -17.23 KN-m

MFB2B1 = WL212 = 18.65 × 3.33212 = +17.23 KN-m

MFB2B3 = -WL212 = -13.47 × 3.28212 = -12.08 KN-m

MFB3B2 = WL212 = 13.47 × 3.28212 = +12.08 KN-m

Fixed end moments in All the columns are zero.

i.e,

MFB1B1՛ = MFB1՛B1 = MFB2B2՛ = MFB2՛B2 = MFB3B3՛ = MFB3՛B3 = 0

Step 2: Rotation Factor R.F = -12 kΣk , k= 4EIL

| Joints | Members | Stiffness (k) | Σk | R.F |
| --- | --- | --- | --- | --- |
| B1 | B1B2 | 1.201EI | 2.682EI | -0.224 |
|  | B1B1՛ | 1.481EI |  | -0.276 |
| B2 | B2B1 | 1.201EI | 3.902EI | -0.154 |
|  | B2B3 | 1.220EI |  | -0.156 |
|  | B2B2՛ | 1.481EI |  | -0.190 |
| B3 | B3B2 | 1.220EI | 2.701EI | -0.226 |
|  | B3B3՛ | 1.481EI |  | -0.274 |

Stiffness (k)      k=4EIL

KB1B2 = 4EI3.33 = 1.201EI

KB1B1՛ = 4EI2.7 = 1.481EI

KB2B1 = 4EI3.33 = 1.201EI

KB2B3 = 4EI3.28 = 1.220EI

KB2B2՛ = 4EI2.7 = 1.481EI

KB3B2 = 4EI3.28 = 1.220EI

KB3B3՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Joints

B1: 1.201EI + 1.481EI = 2.682EI

B2: 1.201EI + 1.220EI + 1.481EI = 3.902EI

B3: 1.220EI + 1.481EI = 2.701EI

Rotation Factor R.F = -12 kΣk

R.FB1B2 = -12 1.2012.682EI = -0.224

R.FB1B1՛ = -12 1.481EI2.682EI = -0.276

R.FB2B1 = -12 1.201EI3.902EI = -0.154

R.FB2B3 = -12 1.220EI3.902EI = -0.156

R.FB2B2՛ = -12 1.481EI3.902EI = -0.190

R.FB3B2 = -12 1.220EI2.701EI = -0.226

R.FB3B3՛ = -12 1.481EI2.701EI = -0.274

Step 3: Displacement Factor D.F = -32 kΣk

| Storey | Members | k | Σk | D.F |
| --- | --- | --- | --- | --- |
| 1 | B1B1՛ | 1.481EI | 4.444EI | -0.5 |
|  | B2B2՛ | 1.481EI |  | -0.5 |
|  | B3B3՛ | 1.481EI |  | -0.5 |

Stiffness (k)      k=4EIL

KB1B1՛ = 4EI2.7 = 1.481EI

KB2B2՛ = 4EI2.7 = 1.481EI

KB3B3՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Storey

∑k = 1.481EI + 1.481EI + 1.481EI = 4.444EI

Displacement Factor D.F = -32 kΣk

D.FB1B1՛ = -32 1.481EI4.444EI = -0.5

D.FB2B2՛ = -32 1.481EI4.444EI = -0.5

D.FB3B3՛ = -32 1.481EI4.444EI = -0.5

Rotation and Displacement Contribution

Cycle 1

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MB1B2 = -0.224 × [-17.23 + 0 + 0] = 3.860

MB1B1՛ = -0.276 × [-17.23 + 0 + 0] = 4.755

MB2B1 = -0.154 × [5.15 + 3.860 + 0] = -1.387

MB2B3 = -0.156 × [5.15 + 3.860 + 0] = -1.405

MB2B2՛ = -0.190 × [5.15 + 3.860 + 0] = -1.712

MB3B2 = -0.226 × [12.08 + (-1.405) + 0] = -2.412

MB3B3՛ = -0.274 × [12.08 + (-1.405) + 0] = -2.925

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MB1B1՛ = MB2B2՛ = MB3B3՛ = -0.5 × (4.755 + (-1.712) + (-2.925)) = -0.059

Cycle 2

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MB1B2 = -0.224 × [-17.23 + (-0.059) + (-1.387)] = 4.183

MB1B1՛ = -0.276 × [-17.23 + (-0.059) + (-1.387)] = 5.154

MB2B1 = -0.154 × [5.15 + (-0.059) + 4.183 + (-2.412)] = -1.057

MB2B3 = -0.156 × [5.15 + (-0.059) + 4.183 + (-2.412)] = -1.070

MB2B2՛ = -0.190 × [5.15 + (-0.059) + 4.183 + (-2.412)] = -1.304

MB3B2 = -0.226 × [12.08 + (-0.059) + (-1.070)] = -2.475

MB3B3՛ = -0.274 × [12.08 + (-0.059) + (-1.070)] = -3.001

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MB1B1՛ = MB2B2՛ = MB3B3՛ = -0.5 × (5.154 + (-1.304) + (-3.001)) = -0.425

Cycle 3

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MB1B2 = -0.224 × [-17.23 + (-0.425) + (-1.057)] = 4.191

MB1B1՛ = -0.276 × [-17.23 + (-0.425) + (-1.057)] = 5.164

MB2B1 = -0.154 × [5.15 + (-0.425) + 4.191 + (-2.475)] = -0.992

MB2B3 = -0.156 × [5.15 + (-0.425) + 4.191 + (-2.475)] = -1.005

MB2B2՛ = -0.190 × [5.15 + (-0.425) + 4.191 + (-2.475)] = -1.224

MB3B2 = -0.226 × [12.08 + (-0.425) + (-1.005)] = -2.407

MB3B3՛ = -0.274 × [12.08 + (-0.425) + (-1.005)] = -2.918

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MB1B1՛ = MB2B2՛ = MB3B3՛ = -0.5 × (5.164 + (-1.224) + (-2.918)) = -0.511

Cycle 4

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MB1B2 = -0.224 × [-17.23 + (-0.511) + (-0.992)] = 4.196

MB1B1՛ = -0.276 × [-17.23 + (-0.511) + (-0.992)] = 5.170

MB2B1 = -0.154 × [5.15 + (-0.511) + 4.196 + (-2.407)] = -0.990

MB2B3 = -0.156 × [5.15 + (-0.511) + 4.196 + (-2.407)] = -1.003

MB2B2՛ = -0.190 × [5.15 + (-0.511) + 4.196 + (-2.407)] = -1.221

MB3B2 = -0.226 × [12.08 + (-0.511) + (-1.003)] = -2.388

MB3B3՛ = -0.274 × [12.08 + (-0.511) + (-1.003)] = -2.895

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MB1B1՛ = MB2B2՛ = MB3B3՛ = -0.5 × (5.170 + (-1.221) + (-2.895)) = -0.527

Step 4: Final End Moments

Mij = FEM + 2×Mij (Near end Rotation) + Mji (Far end Rotation) +M՛՛ (Displacement)

Beams (No Sway M՛՛= 0)

MB1B2 = -17.23 + 2(4.196) + -0.99 = -9.83

MB2B1 = 17.23 + 2(-0.99) + 4.196 = 19.45

MB2B3 = -12.08 + 2(-1.003) + (-2.388) = -16.47

MB3B2 = 12.08 + 2(-2.388) + (-1.003) = 6.30

Column (With Sway M՛՛= (-0.527))

MB1B1՛ = 0 + 2(5.170) + 0 + (-0.527) = 9.81

MB1՛B1 = 0 + 2(0) + 5.170 + (-0.527) = 4.64

MB2B2՛ = 0 + 2(-1.221) + 0 + (-0.527) = -2.97

MB2՛B2 = 0 + 2(0) + (-1.221) + (-0.527) = -1.75

MB3B3՛ = 0 + 2(-2.895) + 0 + (-0.527) = -6.32

MB3՛B3 = 0 + 2(0) + (-2.895) + (-0.527) = -3.42

Equilibrium Check

Joint B1 = MB1B2 + MB1B1՛ = -9.83 + 9.81 = -0.02 ≈ 0

Joint B2 = MB2B1 + MB2B3՛ + MB2B2՛ = 19.45 + (-16.47) + (-	2.97) = 0.01 ≈ 0

Joint B3 = MB3B2 + MB3B3՛ = 6.30 + (–6.32) = –0.02 ≈ 0

CALCULATION BENDING MOMENT

BMB1B2 = WL28 = 18.65 ×3.3328 = 25.85KN-m

BMB2B3 = WL28 = 13.47 ×3.2828 = 18.11KN-m

BENDING MOMENT DIAGRAM

CALCULATION OF REACTIONS

SPAN B1-B2

Length (L) = 3.33m, UDL (W) = 18.65KN/m

Simply Supported Reactions:

Rtotal = WL2 = 18.65 ×3.332 = 31.05KN

Reaction Due to End Moments:

Rmoments = MB2B1- MB1B2)L = 19.45 - 9.833.33 = 2.89KN

Final Reactions:

RB1 = Rtotal – Rmoments = 31.05 – 2.89 = 28.16KN

RB2(Left) = Rtotal + Rmoments = 31.05 + 2.89 = 33.94KN

SPAN B2-B3

Length (L) = 3.28m, UDL (W) = 13.47KN/m

Simply Supported Reactions:

Rtotal = WL2 = 13.47 ×3.282 = 22.10KN

Reaction Due to End Moments:

Rmoments = MB2B3- MB3B2)L = 16.47- 6.303.28 = 3.10KN

Final Reactions:

RB2(Right) = Rtotal + Rmoments = 22.10 + 3.10 = 25.2KN

RB3= Rtotal – Rmoments = 22.10 – 3.10 = 19KN

Total Reactions:

Reaction at Column B1 (RB1) = 28.16KN

Reaction at Column B2 (RB2) = RB2(Left) + RB2(Right) = 33.94 + 25.2 = 59.14KN

Reaction at Column B3 (RB3) = 19KN