Analysis of Building Frames (Using Kani’s Method)

Gasper Kani, a German engineer, developed another distribution procedure based on slope deflection equations. This method is very useful for the analysis of multistorey frames. The greatest advantage of this method is, even if a mistake is committed in distribution in one of the cycles, it converges finally to the correct answer. Even today, many practising engineers who are not familiar with computer methods, use Kani's method for the analysis of 3 to 4 storey building frames.

This method is first explained for structures with fixed ends. Then, the modifications to handle simply supported and overhanging ends are discussed. Analysis of symmetric frames making use of the symmetry is also explained, after which an analysis of general frames is taken up.

Figure 1: Beam Layout

Figure 2: Frames.

Analysis of Frame C1-C2-C3

Step 1: Fixed End Moments

MFC1C2 = -WL212 = -19.02 × 3.33212 = -17.58 KN-m

MFC2C1 = WL212 = 19.02 × 3.33212 = +17.58 KN-m

MFC2C3 = -WL212 = -18.83 × 3.28212 = -16.88 KN-m

MFC3C2 = WL212 = 18.83 × 3.28212 = +16.88 KN-m

Fixed end moments in All the columns are zero.

i.e,

MFC1C1՛ = MFC1՛C1 = MFC2C2՛ = MFC2՛C2 = MFC3C3՛ = MFC3՛C3 = 0

Step 2: Rotation Factor R.F = -12 kΣk , k= 4EIL

| Joints | Members | Stiffness (k) | Σk | R.F |
| --- | --- | --- | --- | --- |
| C1 | C1C2 | 1.201EI | 2.682EI | -0.224 |
|  | C1C1՛ | 1.481EI |  | -0.276 |
| C2 | C2C1 | 1.201EI | 3.902EI | -0.154 |
|  | C2C3 | 1.220EI |  | -0.156 |
|  | C2C2՛ | 1.481EI |  | -0.190 |
| C3 | C3C2 | 1.220EI | 2.701EI | -0.226 |
|  | C3C3՛ | 1.481EI |  | -0.274 |

Stiffness (k)      k=4EIL

KC1C2 = 4EI3.33 = 1.201EI

KC1C1՛ = 4EI2.7 = 1.481EI

KC2C1 = 4EI3.33 = 1.201EI

KC2C3 = 4EI3.28 = 1.220EI

KC2C2՛ = 4EI2.7 = 1.481EI

KC3C2 = 4EI3.28 = 1.220EI

KC3C3՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Joints

C1: 1.201EI + 1.481EI = 2.682EI

C2: 1.201EI + 1.220EI + 1.481EI = 3.902EI

C3: 1.220EI + 1.481EI = 2.701EI

Rotation Factor R.F = -12 kΣk

R.FC1C2 = -12 1.2012.682EI = -0.224

R.FC1C1՛ = -12 1.481EI2.682EI = -0.276

R.FC2C1 = -12 1.201EI3.902EI = -0.154

R.FC2C3 = -12 1.220EI3.902EI = -0.156

R.FC2C2՛ = -12 1.481EI3.902EI = -0.190

R.FC3C2 = -12 1.220EI2.701EI = -0.226

R.FC3C3՛ = -12 1.481EI2.701EI = -0.274

Step 3: Displacement Factor D.F = -32 kΣk

| Storey | Members | k | Σk | D.F |
| --- | --- | --- | --- | --- |
| 1 | C1C1՛ | 1.481EI | 4.444EI | -0.5 |
|  | C2C2՛ | 1.481EI |  | -0.5 |
|  | C3C3՛ | 1.481EI |  | -0.5 |

Stiffness (k)      k=4EIL

KC1C1՛ = 4EI2.7 = 1.481EI

KC2C2՛ = 4EI2.7 = 1.481EI

KC3C3՛ = 4EI2.7 = 1.481EI

Sum of Stiffness ∑k at Storey

∑k = 1.481EI + 1.481EI + 1.481EI = 4.444EI

Displacement Factor D.F = -32 kΣk

D.FC1C1՛ = -32 1.481EI4.444EI = -0.5

D.FC2C2՛ = -32 1.481EI4.444EI = -0.5

D.FC3C3՛ = -32 1.481EI4.444EI = -0.5

Rotation and Displacement Contribution

Cycle 1

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MC1C2 = -0.224 × [-17.58 + 0 + 0] = 3.938

MC1C1՛ = -0.276 × [-17.58 + 0 + 0] = 4.852

MC2C1 = -0.154 × [0.7 + 3.938 + 0] = -0.714

MC2C3 = -0.156 × [0.7 + 3.938 + 0] = -0.723

MC2C2՛ = -0.190 × [0.7 + 3.938 + 0] = -0.881

MC3C2 = -0.226 × [16.88 + (-0.723) + 0] = -3.651

MC3C3՛ = -0.274 × [16.88 + (-0.723) + 0] = -4.427

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MC1C1՛ = MC2C2՛ = MC3C3՛ = -0.5 × (4.852 + (-0.881) + (-4.427)) = 0.228

Cycle 2

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MC1C2 = -0.224 × [-17.58 + 0.228 + (-0.714)] = 4.047

MC1C1՛ = -0.276 × [-17.58 + 0.228 + (-0.714)] = 4.986

MC2C1 = -0.154 × [0.7 + 0.228 + 4.047 + (-3.651)] = -0.204

MC2C3 = -0.156 × [0.7 + 0.228 + 4.047 + (-3.651)] = -0.206

MC2C2՛ = -0.190 × [0.7 + 0.228 + 4.047 + (-3.651)] = -0.251

MC3C2 = -0.226 × [16.88 + 0.228 + (-0.206)] = -3.820

MC3C3՛ = -0.274 × [16.88 + 0.228 + (-0.206)] = -4.631

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MC1C1՛ = MC2C2՛ = MC3C3՛ = -0.5 × (4.986 + (-0.251) + (-4.631)) = -0.052

Cycle 3

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MC1C2 = -0.224 × [-17.58 + (-0.052) + (-0.204)] = 3.995

MC1C1՛ = -0.276 × [-17.58 + (-0.052) + (-0.204)] = 4.923

MC2C1 = -0.154 × [0.7 + (-0.052) + 3.995 + (-3.820)] = -0.127

MC2C3 = -0.156 × [0.7 + (-0.052) + 3.995 + (-3.820)] = -0.128

MC2C2՛ = -0.190 × [0.7 + (-0.052) + 3.995 + (-3.820)] = -0.156

MC3C2 = -0.226 × [16.88 + (-0.052) + (-0.128)] = -3.774

MC3C3՛ = -0.274 × [16.88 + (-0.052) + (-0.128)] = -4.576

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MC1C1՛ = MC2C2՛ = MC3C3՛ = -0.5 × (4.923 + (-0.156) + (-4.576)) = -0.095

Cycle 4

Joint Rotation C.V.  = R.F × ∑FEM+NCV+FCV

MC1C2 = -0.224 × [-17.58 + (-0.095) + (-0.127)] = 3.988

MC1C1՛ = -0.276 × [-17.58 + (-0.095) + (-0.127)] = 4.913

MC2C1 = -0.154 × [0.7 + (-0.095) + 3.988 + (-3.774)] = -0.126

MC2C3 = -0.156 × [0.7 + (-0.095) + 3.988 + (-3.774)] = -0.128

MC2C2՛ = -0.190 × [0.7 + (-0.095) + 3.988 + (-3.774)] = -0.156

MC3C2 = -0.226 × [16.88+ (-0.095) + (-0.128)] = -3.764

MC3C3՛ = -0.274 × [16.88 + (-0.095) + (-0.128)] = -4.564

Displacement Contribution

For All Columns R.V.  = D.F × ∑Rotations Contribution at Columns

MC1C1՛ = MC2C2՛ = MC3C3՛ = -0.5 × (4.913 + (-0.156) + (-4.564)) = -0.096

Step 4: Final End Moments

Mij = FEM + 2×Mij (Near end Rotation) + Mji (Far end Rotation) +M՛՛ (Displacement)

Beams (No Sway M՛՛= 0)

MC1C2 = -17.58 + 2(3.988) + (-0.126) = -9.73

MC2C1 = 17.58 + 2(-0.126) + 3.988 = 21.31

MC2C3 = -16.88 + 2(-0.128) + (-3.764) = -20.9

MC3C2 = 16.88 + 2(-3.764) + (-0.128) = 9.22

Column (With Sway M՛՛= (-0.096))

MC1C1՛ = 0 + 2(4.913) + 0 + (-0.096) = 9.73

MC1՛C1 = 0 + 2(0) + 4.913 + (-0.096) = 4.82

MC2C2՛ = 0 + 2(-0.156) + 0 + (-0.096) = -0.41

MC2՛C2 = 0 + 2(0) + (-0.156) + (-0.096) = -0.25

MC3C3՛ = 0 + 2(-4.564) + 0 + (-0.096) = -9.22

MC3՛C3 = 0 + 2(0) + (-4.564) + (-0.096) = -4.66

Equilibrium Check

Joint C1 = MC1C2 + MC1C1՛ = -9.73 + 9.73 = 0

Joint C2 = MC2C1 + MC2C3՛ + MC2C2՛ = 21.31 + (-20.9) + (-0.41) = 0

Joint C3 = MC3C2 + MC3C3՛ = 9.22 + (-9.22) = 0

CALCULATION BENDING MOMENT

BMC1C2 = WL28 = 19.02 ×3.3328 = 26.36KN-m

BMC2C3 = WL28 = 18.83 ×3.2828 = 25.32KN-m

BENDING MOMENT DIAGRAM

CALCULATION OF REACTIONS

SPAN C1-C2

Length (L) = 3.33m, UDL (W) = 19.02KN/m

Simply Supported Reactions:

Rtotal = WL2 = 19.02 ×3.332 = 31.67KN

Reaction Due to End Moments:

Rmoments = MC2C1- MC1C2L = 21.31 - 9.733.33 = 3.48KN

Final Reactions:

RC1 = Rtotal – Rmoments = 31.67 – 3.48 = 28.19KN

RC2(Left) = Rtotal + Rmoments = 31.67 + 3.48 = 35.15KN

SPAN C2-C3

Length (L) = 3.28m, UDL (W) = 18.83KN/m

Simply Supported Reactions:

Rtotal = WL2 = 18.83 ×3.282 = 30.88KN

Reaction Due to End Moments:

Rmoments = MC2C3- MC3C2L = 20.9- 9.223.28 = 3.56KN

Final Reactions:

RC2(Right) = Rtotal + Rmoments = 30.88 + 3.56 = 34.44KN

RC3= Rtotal – Rmoments = 30.88 – 3.56 = 27.32KN

Total Reactions:

Reaction at Column C1 (RC1) = 28.19KN

Reaction at Column C2 (RC2) = RC2(Left) + RC2(Right) = 35.15 + 34.44 = 69.59KN

Reaction at Column C3 (RC3) = 27.32KN